SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 2- PSEUDOCODE WRITING TASK (ALGORITHM DESIGN)

| Course Code: | CSA12 | Course Title: | Computer Architecture for Multicore Systems |
| --- | --- | --- | --- |
| CO Assessed: | CO5 | Assessment: | ASSESSMENT TOOL2- PSEUDOCODE WRITING TASK (ALGORITHM DESIGN) |
| Weightage: | 30% | Total Marks: | 25 |
| CO5 Statement: | Construct I/O communication mechanisms by comparing memory-mapped and I/O-mapped techniques, interrupt handling (vectored, hardware, software), DMA controllers, and advanced bus interfaces including PCIe, USB, NVMe, and Thunderbolt. |  |  |
| Student Name: | S.Parameshwar | Reg.No.: | 192521044 |
| Department: | B.TECH[IT[ | Date: | 29/08/2026 |

ANNEXURE A

Pseudocode Writing Task (Algorithm Design)

- Write pseudocode to design a DMA-based data transfer algorithm that moves data from an I/O device to memory while handling interrupts and minimizing CPU involvement.

- Develop pseudocode for an interrupt handling system that prioritizes vectored interrupts from multiple devices and dispatches appropriate service routines.

- Write pseudocode to select between memory-mapped I/O and I/O-mapped I/O dynamically based on latency and bandwidth requirements.

- Design pseudocode for a bus arbitration algorithm that manages multiple devices communicating over PCIe, USB, and Thunderbolt interfaces.

- Write pseudocode to implement a hybrid I/O communication mechanism that uses polling for low-speed devices and DMA with interrupts for high-speed devices.

Code of Conduct:

I (Name-S.Parameshwar / Reg No-192521044) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student : Parameshwar S

MARKS ALLOCATION

|  | Problem Understanding (20%) | Algorithm Design (30%) | Use of Control Structures (20%) | Pseudocode Structure (10%) | Completeness (20%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Identifies all inputs, outputs, constraints accurately | Minor missing elements | Partial understanding | Incorrect interpretation |
| Algorithm Design | 30% | Complete, correct, and logically sound solution | Minor logical gaps | Partially correct logic | Incorrect approach |
| Use of Control Structures | 20% | Efficient and appropriate use of loops, conditions, functions/modules | Minor inefficiencies | Limited or basic usage | Incorrect or missing constructs |
| Pseudocode Structure | 10% | Well-structured, clear, consistent format, easy to follow | Mostly clear with minor issues | Difficult to follow in parts | Poorly structured and unclear |
| Completeness | 20% | Handles all cases; optimized approach | Handles most cases; reasonable efficiency | Handles basic cases only | Incomplete or inefficient |

1. DMA-Based Data Transfer from I/O Device to Memory

Aim

To design a DMA-based data transfer algorithm that transfers data directly from an I/O device to memory while minimizing CPU involvement and handling completion interrupts.

Pseudocode

ALGORITHM DMA_IO_TO_MEMORY

BEGIN

INITIALIZE CPU

INITIALIZE I/O DEVICE

INITIALIZE DMA CONTROLLER

SET SOURCE      ← I/O_DEVICE

SET DESTINATION ← MEMORY

SET DATA_SIZE   ← required number of bytes

CONFIGURE DMA CONTROLLER

SET source_address      ← I/O_DEVICE_ADDRESS

SET destination_address ← MEMORY_ADDRESS

SET transfer_count      ← DATA_SIZE

SET transfer_direction  ← IO_TO_MEMORY

ENABLE DMA_INTERRUPT

ENABLE I/O DEVICE

START DMA TRANSFER

WHILE DMA_TRANSFER_NOT_COMPLETED DO

CPU performs other tasks

IF DMA_INTERRUPT_RECEIVED THEN

SAVE CPU STATE

CHECK DMA STATUS

IF TRANSFER_SUCCESSFUL THEN

SET transfer_status ← COMPLETED

CLEAR DMA INTERRUPT

ELSE

SET transfer_status ← ERROR

IDENTIFY ERROR

HANDLE ERROR

END IF

RESTORE CPU STATE

RETURN FROM INTERRUPT

END IF

END WHILE

VERIFY TRANSFERRED DATA

IF data_is_correct THEN

REPORT "DMA Transfer Successful"

ELSE

REPORT "Data Transfer Error"

END IF

END

Working

- CPU initializes the DMA controller.

- Source, destination, transfer size, and direction are configured.

- DMA transfers data directly from the I/O device to memory.

- CPU is free to perform other operations during the transfer.

- After completion, DMA generates an interrupt.

- CPU executes the DMA interrupt service routine.

- Transfer status is checked and the interrupt is cleared.

Advantage: CPU involvement is minimized because the DMA controller performs the actual data transfer.

2. Vectored Interrupt Handling with Priority

Aim

To design an interrupt handling system that prioritizes interrupts from multiple devices and dispatches the correct interrupt service routine (ISR).

Pseudocode

ALGORITHM VECTORED_INTERRUPT_HANDLER

BEGIN

INITIALIZE INTERRUPT CONTROLLER

DEFINE DEVICES:

TIMER

KEYBOARD

NETWORK

DISK

SENSOR

ASSIGN PRIORITY TO EACH DEVICE

ASSIGN INTERRUPT VECTOR TO EACH DEVICE

ENABLE ALL REQUIRED INTERRUPTS

WHILE SYSTEM_IS_RUNNING DO

IF INTERRUPT_REQUEST_RECEIVED THEN

IDENTIFY INTERRUPT_SOURCE

READ INTERRUPT_VECTOR

DETERMINE PRIORITY OF INTERRUPT

IF higher_priority_interrupt_exists THEN

SELECT highest_priority_interrupt

ELSE

SELECT current_interrupt

END IF

SAVE CPU REGISTERS

SAVE PROGRAM_COUNTER

SAVE PROCESSOR_STATUS

DISPATCH INTERRUPT USING VECTOR

SWITCH TO CORRESPONDING ISR

EXECUTE ISR

CLEAR INTERRUPT_FLAG

RESTORE CPU REGISTERS

RESTORE PROGRAM_COUNTER

RESTORE PROCESSOR_STATUS

RETURN FROM INTERRUPT

END IF

END WHILE

END

Example Priority

Priority 1 → Timer

Priority 2 → Network

Priority 3 → Disk

Priority 4 → Sensor

Priority 5 → Keyboard

Example Vector Table

| Device | Interrupt Vector | ISR |
| --- | --- | --- |
| Timer | 001 | Timer_ISR |
| Network | 002 | Network_ISR |
| Disk | 003 | Disk_ISR |
| Sensor | 004 | Sensor_ISR |
| Keyboard | 005 | Keyboard_ISR |

Working

When multiple devices request interrupts, the interrupt controller identifies their priority. The highest-priority interrupt is serviced first. The interrupt vector directly identifies the corresponding ISR, reducing interrupt dispatch time.

Advantage: Vectored interrupts provide fast and efficient interrupt servicing.

3. Dynamic Selection Between Memory-Mapped I/O and I/O-Mapped I/O

Aim

To dynamically select the appropriate I/O addressing method based on latency and bandwidth requirements.

Pseudocode

ALGORITHM SELECT_IO_MAPPING

INPUT:

latency_requirement

bandwidth_requirement

device_type

BEGIN

READ latency_requirement

READ bandwidth_requirement

READ device_type

IF latency_requirement = HIGH THEN

IF bandwidth_requirement = HIGH THEN

SELECT MEMORY_MAPPED_IO

ELSE

SELECT MEMORY_MAPPED_IO

END IF

ELSE IF bandwidth_requirement = HIGH THEN

SELECT MEMORY_MAPPED_IO

ELSE

SELECT IO_MAPPED_IO

END IF

IF SELECTED_MODE = MEMORY_MAPPED_IO THEN

MAP I/O DEVICE REGISTERS INTO MEMORY ADDRESS SPACE

USE NORMAL LOAD/STORE INSTRUCTIONS

CONFIGURE DEVICE

ELSE IF SELECTED_MODE = IO_MAPPED_IO THEN

ASSIGN SEPARATE I/O ADDRESS SPACE

USE I/O-SPECIFIC INSTRUCTIONS

CONFIGURE DEVICE

END IF

MONITOR I/O PERFORMANCE

MEASURE ACTUAL_LATENCY

MEASURE ACTUAL_BANDWIDTH

IF performance_is_below_requirement THEN

RE-EVALUATE I/O MAPPING

IF requirements_have_changed THEN

SELECT appropriate I/O mode again

END IF

END IF

RETURN SELECTED_MODE

END

Selection Logic

IF latency is HIGH

↓

Choose Memory-Mapped I/O

IF bandwidth is HIGH

↓

Choose Memory-Mapped I/O

IF latency and bandwidth requirements are LOW

↓

I/O-Mapped I/O can be selected

Working

Memory-mapped I/O places device registers in the normal memory address space and allows normal load/store instructions. It is generally convenient when frequent and fast access is required.

I/O-mapped I/O uses a separate I/O address space and dedicated I/O instructions. It can be useful when the architecture supports a separate I/O space and conserving memory address space is important.

4. Bus Arbitration for PCIe, USB, and Thunderbolt

Aim

To design a bus arbitration algorithm that manages communication requests from multiple devices using PCIe, USB, and Thunderbolt interfaces.

Pseudocode

ALGORITHM BUS_ARBITRATION

BEGIN

INITIALIZE PCIe_CONTROLLER

INITIALIZE USB_CONTROLLER

INITIALIZE THUNDERBOLT_CONTROLLER

DEFINE REQUEST_QUEUE

WHILE SYSTEM_IS_RUNNING DO

CHECK PCIe_REQUEST

CHECK USB_REQUEST

CHECK THUNDERBOLT_REQUEST

IF PCIe_REQUEST = TRUE THEN

ADD PCIe_REQUEST TO REQUEST_QUEUE

END IF

IF USB_REQUEST = TRUE THEN

ADD USB_REQUEST TO REQUEST_QUEUE

END IF

IF THUNDERBOLT_REQUEST = TRUE THEN

ADD THUNDERBOLT_REQUEST TO REQUEST_QUEUE

END IF

FOR EACH REQUEST IN REQUEST_QUEUE DO

CALCULATE PRIORITY BASED ON:

DATA_SIZE

LATENCY_REQUIREMENT

DEVICE_PRIORITY

REAL_TIME_REQUIREMENT

END FOR

SELECT HIGHEST_PRIORITY_REQUEST

IF selected_request = PCIe THEN

GRANT PCIe_ACCESS

TRANSFER DATA

RELEASE PCIe_ACCESS

ELSE IF selected_request = USB THEN

GRANT USB_ACCESS

TRANSFER DATA

RELEASE USB_ACCESS

ELSE IF selected_request = THUNDERBOLT THEN

GRANT THUNDERBOLT_ACCESS

TRANSFER DATA

RELEASE THUNDERBOLT_ACCESS

END IF

REMOVE COMPLETED_REQUEST FROM REQUEST_QUEUE

CHECK FOR ERRORS

IF ERROR_DETECTED THEN

HANDLE ERROR

RETRY OR REPORT FAILURE

END IF

END WHILE

END

Priority Example

Real-time request

↓

High-bandwidth request

↓

Low-latency request

↓

Normal request

↓

Background request

Working

- Controllers monitor requests from PCIe, USB, and Thunderbolt devices.

- Requests are placed into a queue.

- Each request is assigned a priority.

- The arbitration logic selects the highest-priority request.

- Access is granted to the selected interface.

- Data transfer takes place.

- After completion, the resource is released.

- The next request is processed.

Note: In real systems, PCIe, USB, and Thunderbolt have different protocol-specific architectures and arbitration mechanisms. The pseudocode represents a high-level system-level arbitration policy, not a replacement for their actual protocol controllers.

- Hybrid I/O Communication Mechanism

Aim

To design a hybrid I/O mechanism that uses polling for low-speed devices and DMA with interrupts for high-speed devices.

Pseudocode

ALGORITHM HYBRID_IO_COMMUNICATION

BEGIN

INITIALIZE ALL I/O DEVICES

FOR EACH DEVICE DO

DETERMINE DEVICE_SPEED

DETERMINE DATA_SIZE

DETERMINE LATENCY_REQUIREMENT

IF DEVICE_SPEED = LOW AND

DATA_SIZE = SMALL THEN

SET MODE ← POLLING

ELSE

SET MODE ← DMA_WITH_INTERRUPT

END IF

END FOR

WHILE SYSTEM_IS_RUNNING DO

FOR EACH DEVICE DO

IF MODE = POLLING THEN

CHECK DEVICE_STATUS

IF DEVICE_READY THEN

READ DATA FROM DEVICE

PROCESS DATA

CLEAR DEVICE_STATUS

END IF

ELSE IF MODE = DMA_WITH_INTERRUPT THEN

CONFIGURE DMA_CONTROLLER

SET SOURCE_ADDRESS

SET DESTINATION_ADDRESS

SET TRANSFER_SIZE

ENABLE DMA_INTERRUPT

START DMA_TRANSFER

CPU CONTINUES OTHER TASKS

IF DMA_INTERRUPT_RECEIVED THEN

SAVE CPU STATE

CHECK DMA_STATUS

IF TRANSFER_SUCCESSFUL THEN

PROCESS TRANSFERRED DATA

ELSE

HANDLE DMA_ERROR

END IF

CLEAR INTERRUPT

RESTORE CPU STATE

END IF

END IF

END FOR

END WHILE

END

Example Device Classification

| Device Type | Data Rate | Selected Method |
| --- | --- | --- |
| Keyboard | Low | Polling |
| Simple Sensor | Low | Polling |
| Mouse | Low/Medium | Polling |
| Network Interface | High | DMA + Interrupt |
| SSD/Storage | Very High | DMA + Interrupt |
| High-speed Camera | High | DMA + Interrupt |

Working

Advantages

- Polling is simple and suitable for low-speed devices.

- DMA reduces CPU involvement for large/high-speed transfers.

- Interrupts notify the CPU only when DMA operations require attention.

- The hybrid approach improves overall CPU utilization and I/O performance.

- Different devices can use the communication method most appropriate for their requirements.