SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 1- INDUSTRY PROBLEM SOLVING TASK

| Course Code: | CSA12 | Course Title: | Computer Architecture for Multicore Systems |
| --- | --- | --- | --- |
| CO Assessed: | CO4 | Assessment: | ASSESSMENT TOOL1- INDUSTRY PROBLEM SOLVING TASK |
| Weightage: | 30% | Total Marks: | 25 |
| CO4 Statement: | Implement hierarchical memory systems by differentiating cache levels (L1, L2, L3), virtual memory with paging, and advanced memory technologies (DRAM, SRAM, NAND Flash, MRAM, RRAM) based on latency, bandwidth, and throughput characteristics. |  |  |
| Student Name: | Parameshwar S | Reg.No.: | 192521044 |
| Department: | B.TECH[IT] | Date: | 27/08/2026 |

ANNEXURE A

INDUSTRY PROBLEM SOLVING TASK

- Smartphone Performance Optimization                                 
 A mobile manufacturer observes lag while switching between apps. Analyze the role of L1, L2, L3 cache and DRAM and propose a hierarchical memory redesign to reduce latency and improve throughput.

- Cloud Server Memory Bottleneck
 A cloud service provider experiences high latency in database queries. Compare cache hierarchy vs virtual memory paging and recommend improvements using appropriate memory technologies.

- Autonomous Vehicle Data Processing
 An autonomous vehicle must process sensor data in real time. Evaluate SRAM, DRAM, and MRAM and design a memory hierarchy that maximizes bandwidth and minimizes latency.

- AI Inference Edge Device
 An edge AI device needs fast model loading with low power consumption. Compare NAND Flash, DRAM, and RRAM and recommend an optimized hierarchical memory structure.

- High-Performance Gaming Console
 A gaming console suffers frame drops during large scene loading. Analyze L3 cache vs DRAM throughput and propose memory hierarchy enhancements.

Code of Conduct:

I  (Name-S.Parameshwar / Reg No-192521044) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student : PARAMESHWAR S

MARKS ALLOCATION

|  | Understanding of Industry Problem (20%) | Application of Engineering Concepts (25%) | Solution Design (25%) | Use of Tools (15%) | Analysis (10%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understanding of Industry Problem | 20% | Comprehensive understanding of real-world problem and constraints | Good understanding with minor gaps in requirements | Basic understanding; some requirements unclear | Poor understanding; misinterprets problem context |
| Application of Engineering Concepts | 25% | Applies advanced and relevant concepts effectively to solve the problem | Applies appropriate concepts with minor errors | Limited application of concepts | Incorrect or no application of concepts |
| Solution Design | 25% | Innovative, practical, and feasible solution aligned with industry standards | Logical and feasible solution with minor gaps | Basic solution with limited feasibility | Unclear or impractical solution |
| Use of Tools | 15% | Effective use of modern tools, technologies, and real data | Appropriate use of tools with correct implementation | Limited or inconsistent use of tools | Minimal or incorrect use of tools |
| Analysis | 10% | Thorough analysis with validated results and performance evaluation | Good analysis with reasonable validation | Basic analysis with limited validation | Poor or no analysis |

1. Smartphone Performance Optimization

Problem Analysis

The mobile manufacturer observes lag while switching between applications. The main reason is not simply insufficient RAM; it is the increasing gap between processor speed and memory access speed.

During app switching, the processor frequently accesses application instructions, UI data, background processes, and recently used memory pages. If required data is not present in the faster cache levels, the processor must access slower memory.

Role of Different Memory Levels

| Memory | Main Role | Latency | Capacity | Best Use |
| --- | --- | --- | --- | --- |
| L1 Cache | Stores immediately required instructions/data | Very Low | Very Small | Active app execution |
| L2 Cache | Stores frequently reused data missed by L1 | Low | Small/Medium | Working data |
| L3 Cache | Shared cache for multiple CPU cores | Moderate | Larger | Cross-core/shared data |
| DRAM | Main working memory | High compared with cache | Large | Applications/background apps |

Proposed Hierarchical Redesign

Unique Optimization

Instead of treating every application equally, the operating system can classify applications into:

- Hot apps – currently active or frequently switched to

- Warm apps – recently used applications

- Cold apps – rarely used applications

Frequently accessed data from hot apps should remain available in cache and DRAM, while cold applications can be compressed or moved out of the active working set.

Expected Improvement

The redesigned hierarchy reduces:

- Cache miss penalties

- DRAM accesses

- App-switch latency

- CPU idle time

It also improves throughput because the processor spends more cycles executing instructions rather than waiting for memory.

Result

A cache-aware and workload-aware memory hierarchy can significantly improve smartphone responsiveness during multitasking while maintaining the same physical DRAM capacity.

2. Cloud Server Memory Bottleneck

Problem Analysis

The cloud provider experiences high database-query latency. Database systems repeatedly access indexes, rows, transaction metadata, and frequently used query results.

Two mechanisms must be distinguished:

Cache hierarchy reduces processor-to-memory access latency.

Virtual memory paging manages memory capacity by moving pages between DRAM and secondary storage.

Therefore, paging should not be used as a replacement for cache optimization.

Cache Hierarchy vs Virtual Memory Paging

| Feature | Cache Hierarchy | Virtual Memory Paging |
| --- | --- | --- |
| Main Purpose | Reduce access latency | Extend usable memory |
| Managed By | Hardware | OS + hardware |
| Fastest Storage | SRAM cache | DRAM |
| Slow Path | DRAM | SSD/storage |
| Main Problem | Cache miss | Page fault |
| Database Benefit | Faster repeated CPU accesses | Supports large workloads |

Proposed Architecture

Unique Improvement: Database-Aware Memory Tiering

Instead of allowing frequently accessed database pages to be randomly evicted, the server can classify pages according to access frequency.

Hot data → DRAM

Frequently executed instructions → CPU cache

Warm data → compressed memory

Cold/inactive pages → NVMe SSD

This reduces unnecessary page faults.

Recommended Technologies

- High-speed SRAM for CPU caches

- High-capacity DRAM for database working sets

- NVMe SSD as secondary storage

- Memory compression to increase effective DRAM capacity

Performance Evaluation

If a database repeatedly accesses the same index:

Without optimization:

CPU → Cache Miss → DRAM → Possible Page Fault → SSD

With optimization:

CPU → L1/L2/L3 → DRAM

The second path avoids expensive storage access.

Result

The best solution is not simply increasing DRAM. A cache-aware database memory hierarchy combined with intelligent virtual-memory/page management reduces database-query latency and improves server throughput.

3. Autonomous Vehicle Data Processing

Problem Analysis

An autonomous vehicle continuously receives data from cameras, LiDAR, radar, ultrasonic sensors and other vehicle systems. Unlike a normal application, delayed processing can affect the vehicle's response time.

Therefore, the design objective is:

Minimum latency + maximum bandwidth + predictable access time.

SRAM vs DRAM vs MRAM

| Technology | Speed | Volatility | Density | Suitable Role |
| --- | --- | --- | --- | --- |
| SRAM | Extremely Fast | Volatile | Low | Real-time processing/cache |
| DRAM | Fast | Volatile | High | Sensor data buffer |
| MRAM | Fast | Non-volatile | Medium | Persistent/critical data |

Proposed Autonomous Vehicle Memory Hierarchy

Unique Design: Priority-Based Sensor Memory

Sensor information should not be treated equally.

For example:

Emergency braking data → SRAM

Current camera/LiDAR processing frames → DRAM

Critical configuration/calibration/state data → MRAM

This creates a priority-aware memory system.

Why SRAM for Real-Time Data?

SRAM has very low latency, so it is appropriate for the smallest and most time-critical working sets.

Why DRAM?

The vehicle's AI and sensor-processing algorithms need much more memory than SRAM can economically provide. DRAM therefore acts as the main high-capacity working memory.

Why MRAM?

MRAM provides non-volatile storage characteristics with relatively fast access, making it useful for information that should survive power interruption.

Result

The proposed SRAM–DRAM–MRAM hierarchy separates urgent data, high-volume data and persistent data, maximizing processing bandwidth while minimizing the latency of safety-critical operations.

4. AI Inference Edge Device

Problem Analysis

An edge AI device needs to load machine-learning models quickly while consuming little power.

The challenge is that:

- NAND Flash provides high capacity but slower access.

- DRAM provides fast working memory but is volatile.

- RRAM can provide non-volatile memory characteristics and can be useful for AI-oriented memory architectures.

Memory Comparison

| Technology | Speed | Power | Volatile? | Main Application |
| --- | --- | --- | --- | --- |
| NAND Flash | Moderate/Slow | Low when idle | No | Model storage |
| DRAM | Fast | Higher | Yes | Active inference |
| RRAM | Fast potential access | Low | No | AI model/weight storage |

Proposed Hierarchy

Unique Optimization: Model Tiering

Instead of loading the entire AI model every time:

- Store the complete model in NAND Flash.

- Keep frequently used model components in RRAM.

- Move the currently executing model into DRAM.

- Keep intermediate activations in DRAM.

- Replace only required model sections.

This reduces unnecessary data movement.

Example

NAND Flash

↓

Complete AI Models

↓

RRAM

Frequently used weights

↓

DRAM

Current inference model

↓

AI Accelerator

Benefits

- Faster model startup

- Reduced NAND accesses

- Lower energy consumption

- Better inference throughput

- Efficient use of limited edge-device memory

Result

The NAND → RRAM → DRAM hierarchy combines the capacity of NAND, persistence and AI-oriented potential of RRAM, and high-speed working capability of DRAM to create a power-efficient edge AI architecture.

5. High-Performance Gaming Console

Problem Analysis

The gaming console experiences frame drops while loading large scenes. Modern game scenes contain:

- Textures

- 3D models

- Lighting information

- Animation data

- Physics data

- Audio assets

When these assets cannot reach the processor/GPU quickly enough, the rendering pipeline waits, producing frame-time spikes.

L3 Cache vs DRAM

| Feature | L3 Cache | DRAM |
| --- | --- | --- |
| Capacity | Smaller | Much Larger |
| Latency | Lower | Higher |
| Technology | SRAM | DRAM |
| Cost per bit | High | Lower |
| Best Use | Frequently reused data | Large game assets |
| Main Limitation | Limited capacity | Higher latency |

Proposed Gaming Memory Hierarchy

Unique Solution: Predictive Scene Prefetching

The console should predict which game assets will be needed next.

For example:

Player moving toward Area B

↓

Game Engine predicts Area B

↓

Textures + Models + Lighting

prefetched from storage

↓

DRAM

↓

Frequently reused assets → L3

↓

CPU/GPU rendering

Instead of waiting until the player enters the next area, the system loads assets before they are required.

L3 Optimization

The L3 cache should prioritize:

- Frequently reused game-engine instructions

- Physics data

- AI state

- Frequently accessed rendering metadata

Large textures should generally remain in larger memory rather than attempting to place everything in L3.

Performance Goal

The objective is not merely increasing L3 size. The important metric is:

Frame-time consistency.

Poor design:

16 ms → 17 ms → 45 ms → 16 ms

↑

Frame drop

Optimized design:

16 ms → 16 ms → 17 ms → 16 ms

A predictive prefetch system can reduce large memory stalls and improve frame-time stability.

Result

A cache-aware + predictive-prefetch memory hierarchy allows frequently reused data to remain close to the processor while large scene assets are streamed into DRAM ahead of demand, reducing frame drops and improving gaming performance.

Comparison

| Industry Problem | Primary Bottleneck | Proposed Key Idea | Main Benefit |
| --- | --- | --- | --- |
| Smartphone | App-switch latency | Hot/Warm/Cold app tiering | Faster multitasking |
| Cloud Server | Database memory latency | Database-aware page tiering | Lower query latency |
| Autonomous Vehicle | Real-time sensor processing | Priority-based SRAM/DRAM/MRAM | Deterministic response |
| Edge AI | Model loading + power | NAND/RRAM/DRAM model tiering | Faster, energy-efficient inference |
| Gaming Console | Large scene loading | Predictive asset prefetching | Fewer frame drops |

Conclusion

The major design principle across all five industries is that one memory technology cannot simultaneously provide minimum latency, maximum capacity, low power and high bandwidth. Therefore, an efficient computer architecture uses a hierarchical memory system, placing the most time-critical and frequently accessed information in fast memory while keeping large or less frequently accessed information in higher-capacity memory.

The proposed solutions go beyond simply increasing memory size. They use workload-aware placement, priority-based buffering, intelligent paging, model tiering and predictive prefetching. This makes the architectures practical for modern smartphones, cloud servers, autonomous vehicles, edge AI systems and gaming consoles.

Top of Form

Bottom of Form